SAT-17M2 Handheld Fusion Splicer – Ultra-Lightweight, Fast, Field-Ready

Description

The SAT-17M2 Handheld Fusion Splicer is an ultra-lightweight, portable splicing device developed by AITELONG specifically for on-site fiber optic installation and maintenance. Weighing only 1.3 kg with battery, it employs a precise cladding alignment technique and delivers typical splice losses of 0.04 dB for single-mode fiber, 0.03 dB for multi-mode fiber, 0.04 dB for dispersion-shifted fiber, and 0.05 dB for non-zero dispersion-shifted fiber. With a typical splicing time of 7 seconds and a heating time of ≤15 seconds (for 40 mm protective sleeves), the unit features a 4.3-inch LCD touchscreen and integrates an optical power meter (OPM) and a visual fault locator (VFL), eliminating the need for separate test instruments in the field. Its windproof, dustproof, and waterproof design ensures reliable operation in a wide range of demanding environmental conditions.

The SAT-17M2 is widely used in FTTH deployments, access network construction, campus networks, and routine fiber optic maintenance. Equipped with a 3-in-1 fiber holder, it is powered by a 3000 mAh rechargeable battery that supports approximately 120 splicing and heating cycles per full charge, with a battery lifespan of 300–500 charge cycles and replaceable electrodes rated for 4,000 typical splices, making it well suited for extended field operations.

As a specialized manufacturer of optical communication test instruments with over 20 years of industry experience, AITELONG is committed to delivering high-quality fiber optic splicing and testing solutions to customers worldwide. The SAT-17M2 has become a trusted choice for telecom engineering companies and field maintenance teams, thanks to its ultra-lightweight portability, low splice loss, touchscreen operation, and built-in test functions. We provide professional technical support and comprehensive after-sales service to ensure that our customers receive timely and reliable assistance throughout the life of the product.

Key Features

Ultra-light weight: 1.3kg with battery

Low splice loss

Fast splicing: 7s (typical)

Fast heating: 15s (for 40mm protective sleeves)

All-in-one fiber holder, compatible with bare fiber, pigtail, jumper and other fibers

Auto-start heating (built-in sensor)

Dual operation: buttons and touch screen

Multi-function: built-in VFL and OPM

Locking function by password for easy machine management

Wind protective, dustproof, water-proof

General Specification

| Alignment Method | Precise cladding, manual alignment |
| --- | --- |
| Applicable Fiber | ITU-T G.651~653、G.655、G.657 fiber, pigtail, drop cable, etc. |
| Fiber Diameter | Cladding: 80~150 μ m; Coating: 0.1~3mm |
| Typical Splice Loss | 0.04dB(SMF); 0.03dB(MMF); 0.04dB(DSF); 0.05dB(NZDSF) |
| Splicing Time | 7s (Typical) |
| Heating Time | ≤ 15s (40mm protective sleeves) |
| Heat Shrink Sleeve | 60mm, 40mm and 20mm etc |
| Display | 4.3 inch LCD touch screen |
| Splicing Record | 30,000pcs data & 200 images |
| Battery Capacity | 3000mAh (120 splicing and heating cycles) |
| Battery Life | 300~500 recharge cycles, replaceable |
| Electrode Life | Typical 4000 times, replaceable |
| Lighting | Built-in lights |
| Operating Environment | Temp: -10~50℃; Humidity: 0~95%RH; Altitude: 0~5000m |
| User Interface | GUI graphical user interface |
| Power Supply | AC: 100~240V, 50/60Hz; DC: 10~15V |
| Interface | USB |
| VFL and OPM | Calibration wavelengths: 1300/1310/1490/1550/1625/1650 (nm) |
| Weight | 1.3kg with battery |

Standard Packing List

| No. | Name & Description | Quantity |
| --- | --- | --- |
| 1 | Splicing Machine Host | 1pc |
| 2 | High Precision Fiber Cleaver | 1pc |
| 3 | Carry Case | 1pc |
| 4 | Power Cord | 1pc |
| 5 | Power Adapter | 1pc |
| 6 | Fiber Stripper | 1pc |
| 7 | Drop Cable Stripper | 1pc |
| 8 | Spare Electrodes | 1pair |
| 9 | Alcohol Bottle | 1pc |
| 10 | Cooling Tray | 1pc |
| 11 | User Manual | 1pc |