SAT-18EXB Mini OTDR

Applicable Scenarios

1. Access Network (FTTx/FTTH)

2. Metro Network (Short to Medium Span)

3. Data Center & Equipment Room

4. Field Deployments & Emergency Restoration

5. Industry Specific Networks (Power, Railway, Metro)

6. Pre-construction Survey & Preliminary Acceptance

Key Features

- Suitable for FTTx and other access network, test fiber’s length, loss, breakpoint etc.

- 3.5“colorful LCD screen, utral-light design for easily carrying,

- 8 in 1: OTDR, OLS, OPM, VFL, iOTA(Event Map),RJ Sequence, RJ length and LED Lighter.

- Universal connector: FC/SC/ST/LC for choice.

- Large internal storage with 600 results, and extensible TF card, USB wire can upload the testing results to PC for printing.

- Built-in 4000mAh li-ion rechargeable battery, can charge by any USB charger, suitable for long-term field working.

General Specification

| Screen | 3.5“Colorful LCD screen |
| --- | --- |
| Connect Port | FC/UPC(SC、ST available) |
| Com. Port | Micro USB*1 |
| Results Storage | Internal：≥600；External：TF Card; File type: SOR Standard file format |
| Battery Life | stand by>7.5h； testing >4h |
| Power Supply | 3.7V, 4000mAh lithium battery; Power adapter:5VDC, 2A. |
| Working Environments | Working temperature: 0~+50℃，<95%RH |
| Dimensions/Weight | 173mm*82mm*37mm, <350g |

Technical Specification

| Dynamic Range |  | 24/22dB |
| --- | --- | --- |
| Wavelength |  | 1310nm/1550nm±20nm |
| Fiber Type |  | SM (G.652) |
| Event Dead Zone |  | 3m |
| Attenuation Dead Zone |  | 8m |
| Pulse Width |  | 3ns to10us |
| Distance Range |  | 500m to 60km |
| Measurement Accuracy |  | ± (1m+sample space+0.005%*measurement distance) |
| Linearity |  | ±0.2 dB/dB |
| Sample Points |  | 16k~128k |
| Sample Resolutions |  | 0.05m~8m |
| IOR |  | ±3dB |
| VFL | Output Power | 10mW |
|  | Wavelength | 650±20nm |
|  | Working Mode | CW/1Hz/2Hz |
|  | Connect Port | FC/SC/ST |
| Power Meter | Wavelength | 800nm to 1700 nm |
|  | Calibrated wavelength | 850/980/1300/1310/1490/1550/1625 nm |
|  | Measurement range | -50 to +26 dBm |
| Laser Source | Working Wavelength | 1310nm/1550nm±20nm |
|  | Output Power | ≥-5dBm |
|  | Frequency | 270Hz /330Hz /1kHz /2kHz |
| RJ45 | Testing Distance | 300m |

Packing List

| No. | Description | Qty | Remarks |
| --- | --- | --- | --- |
| 1 | Main Unit | 1 | With battery |
| 2 | Power Adapter | 1 |  |
| 3 | Data Cable | 1 |  |
| 4 | CD (Including analysis software & user manual) | 1 |  |
| 5 | Operation Manual | 1 |  |
| 6 | SC/UPC Connector | 1 |  |
| 7 | Test Report | 1 |  |
| 8 | Certificate of Conformity / After-sales Warranty Card | 1 |  |
| 9 | Cleaning Wipes | 10 |  |
| 10 | Instrument Carry Case |  |  |