SAT-18EXC Palm OTDR

Palm OTDR is a compact, portable test instrument purpose-built for short-to-medium distance fiber testing, striking an ideal balance between performance and portability:

Applicable Scenarios

Access Network (FTTx/FTTH)

•	Covers 10 30km links from OLT to subscriber, including splitters and drop cables.

•	Tasks: Link verification, splitter loss testing, drop fiber break location, quality spot checks.

II. Metro Network (Short to Medium Span)

•	Quick health checks on 10 20km ring or mesh spans.

•	Tasks: Base station fiber inspection, loss re measurement, pre /post cutover comparison.

III. Data Center & Equipment Room

•	Tests short distance internal links (hundreds of meters to a few kilometers).

•	Tasks: Patch cord/pigtail quality check, MPO link event detection, rapid relocation verification.

IV. Field Deployments & Emergency Restoration

•	Portable, quick response testing for military and disaster recovery comms.

•	Tasks: Link continuity check, outdoor break location, temporary route validation.

V. Industry Specific Networks (Power, Railway, Metro)

•	For OPGW/ADSS, railway/metro wayside fibers (10 30km spans) in tower or tunnel environments.

•	Tasks: Splice inspection, tunnel troubleshooting, routine periodic inspection.

VI. Pre Construction Survey & Preliminary Acceptance

•	Rapid screening of cable segments before formal certification.

•	Tasks: Cable length/loss spot check, laid segment quality screening, splice validation before closure

Features

1. Compact & Lightweight, One-Hand Operation

- Palm-sized design (typically comparable to a smartphone), weighing approximately 300-500g including battery, enabling true one-hand operation. Ideal for tower climbing, manhole work, and cramped equipment rooms — significantly reducing carrying burden and improving operational convenience.

2. 24dB Dynamic Range Covers Mainstream Short-to-Medium Distances

- 24dB dynamic range is sufficient to test 10-40km of single-mode fiber (depending on link loss coefficient), clearly identifying splices, connectors, bends, and fiber breaks within that range, meeting the vast majority of access network and metro-edge testing requirements.

3. Extremely Simple Operation, Quick to Master

- One-button auto-test with built-in intelligent event diagnosis automatically identifies event types (breaks, high-loss splices, macro-bends, etc.) and marks distance and loss values. Operators do not need in-depth OTDR parameter expertise, significantly reducing training costs and user learning curve.

4. Fast Boot-Up, Ideal for High-Frequency Short-Duration Tests

- Ready to use immediately after power-on, with single test time typically 15-30 seconds. For inspection and troubleshooting scenarios requiring frequent "quick-check" measurements, it is far more efficient than large benchtop or high-precision OTDRs.

5. Long Battery Life, Suited for Field Operations

- Built-in high-capacity Li-ion battery supports 8+ hours of continuous testing or hundreds of single measurements per charge. USB-C or power-bank recharging capability meets full-day field operation needs.

6. Touchscreen + Physical Buttons for Dual Operation

- Combines intuitive touchscreen interaction with reliable physical button operation — buttons remain fully functional even when wearing gloves or in rain/snow conditions.

7. Convenient Data Management

- Supports USB data transfer or Bluetooth/Wi-Fi wireless export; test traces can be saved in standard SOR format, compatible with mainstream analysis software for easy report generation.

Technical Specification

| Dynamic Range | 24/22dB |
| --- | --- |
| Wavelength | 1310/1550nm |
| Fiber Type | SM |
| Event Dead Zone | 2m |
| Attenuation Dead Zone | 4m |
| Pulse Width | 5ns,10ns,20ns,50ns,100ns,300ns,1.0ms,2.5ms,10ms, 20ms |
| Distance Range | 0.5 to 80km |
| Loss Test Accuracy | ±0.05dB/ dB |
| Linearity | ±5% |
| Screen size | 3.5 inch |
| storage | 500 |
| Standard port | FC/SC/ST/LC optional |
| Battery | Continuous working>10 hours |
| Size | L*W*H：145mm×78mm×30mm |
| Weight | 300g |
| VFL (optional) | Wavelength: 650±20nm; Output Power:10mW Working mode:CW/1Hz/2Hz; Standard connector: FC/UPC |
| OPM (Optional) | Calibrated wavelength: 850,980,1270,1300,1310,1490,1550,1577,1625nm Dynamic range: -70 to +6 dBm; -50 ~ +26 (optional) Detector type: InGaAs |
| Laser source (Optional) | Wavelength(nm)：Same with OTDR; Output: ≥ -7dBm |