SAT-1CJ E1/Datacom and Jitter Tester for digital transmission system

Introduction and applications:

SAT-1CJ is a multi-functional and full- featured digital transmission system test device, designed for the installation test, engineering check and acceptance, daily maintenance of digital networks, mainly performing channel test, alarm analysis, fault finding and signaling analysis. In addition, this instrument further provides various protocol converters with one-way and bi-directional bit error test function. These capabilities make it ideal for field use.

Functions:

- E1 test

	Service-interrupted error testing

	On-line service error testing

	Framed and unframed signals generation and reception

	2Mbit/s unframed error performance testing

	2Mbit/s framed N×64Kbit/s channel error testing

	Bit error, coding error, frame error, CRC error and E-bit error testing

	Signal loss alarm, AIS alarm, framed remote alarm, multi-framed remote alarm, out-of-frame, and pattern loss alarm

	Frequency offset transmitting

	Voice channel signal level and frequency testing

	Pattern slip testing

	Pass through testing

	Audio frequency testing

	Voice monitoring

	Signal state display, Voice channel content display, Voice channel busy / idle indication

	Alarm and error histogram analysis

	Time slot content analysis, drop and insert signal on each time slot

	Framed content analysis

	G. 821/G. 826/M. 2100 performance analysis

	Multi errors and alarms inserting

	Three input modes (terminating, bridging and monitoring)

	Provides two clock options (internal and picking-up)

	Loop delay test

	Clock slip testing

	Send clock offset

	Automatic protection switching time testing (APS)

	Two line of 2MBit/s testing, two-way monitoring

- Datacom test (only available for model SAT-1CS and SAT-1CJ)

	Includes V.24/V.28/RS232, V.35, V.36/RS-449, X.21, RS-485, RS422, EIA-530, EIA-530A

	SYNCH and ASYNCH testing

	DTE and DCE emulation

	Bit code testing

	Pattern slip testing

	Signal loss alarm

	Line signal frequency testing

	Loop delay testing

	Automatic protection switching time testing(APS)

	G.821, M2100 service-interrupted error testing

3.  G.703 CO measurement

Co-directional 64Kbit/s (Optional)

	Service-interrupted error testing

	Bit code testing

	Pattern slip testing

	Signal loss, AIS alarm

	Line signal frequency testing

	Loop delay testing

	Automatic protection switching time testing(APS)

	G.821, M.2100 error performance testing

- Protocol converter measurement

2M-Datacom SYNC 64k or N×64kb/s BER testing

2M-G.703 CO SYNC 64kb/s BER testing

Frequency and offset measurement

Handshaking signals monitoring

2M frame data and alarm monitoring

- Other functions

Real-time clock

Test pattern: PRBS, Fixed Code and 16-BIT User Word Error injection: Single and Fixed Rate

Manual and auto-timer measurement

ITU-T G.821, G.826, and M.2100 performance analysis

- Jitter Test (O.171, O. 172)

Key Features

Handheld, rugged design and easy-to-use;

Smart auto configuration feature

High resolution backlit 5 inch color TFT LCD screen

Large capacity battery support 6 hours work

Save/Recall of up to 7 user-defined setups and 99 sets of results

Test results uploaded, conserved and printed by PC Manager software.

Particular history and real-time LED indicator design

Automatically power on/off testing by programmable timer

Alarm and Histogram analysis for troubleshooting

Ordering Information (MODELS)

| Model No. | SAT-1AS | SAT-1CS | SAT-1CJ |
| --- | --- | --- | --- |
| E1 Measurement | Yes | Yes | Yes |
| Datacom Measurement | No | Yes | Yes |
| Audio/Listen | Yes | Yes | Yes |
| Frequency Deviation | No | Yes | Yes |
| Pulse Mask | No | Yes | Yes |
| Co directional 64K | Optional | Optional | NO |
| Jitter | Optional | Optional | Yes |
| Frame Relay | Optional | Optional | Yes |
| AC Power Adapter | AC220V 50Hz/DC 9V 1A |  |  |
| Rechargeable Batteries | 5*1.2V AA Ni MH batteries |  |  |
| Operating/Storage Temperature | 0oC~50oC/-20~+70oC |  |  |
| Humidity | 5%~95% non-condensing |  |  |
| Dimensions | 200*160*42mm |  |  |
| Weight (Host) | 2kg |  |  |

Technique Specification

TX：

● Output interface：75 Ohm balanced, 120 Ohm balanced, conforming to G.703 requirements

● Signal structure：

Unframed

Frame (PCM30, PCM31, PCM30CRC, PCM31CRC), conforming to G.704 n×64 Kbit/s structure

● Line code：HDB3 or AMI

● Signal rate：2048 kbit/s

● Internal clock accuracy：≤±10ppm

● Clocking：Internal, external and distill

● Output intrinsic twittering：<0.05UI(20Hz-100kHz)

● Test pattern： False random sequence 2E6-1,2E9-1,2E11-1,2E15-1, 2E20-1, 2E23-1, conforming to ITU-T0.151, man code 8 bit

● Free TS pattern:8bit man mode

● Code error insertion：

FAS ERR：once, continuous twice, continuous 3 times, continuous 4 Times

BIT ERR： single  1E-2～1E-7

● Pattern slip：PAT slip

RX：

● Signal input impedance：

terminal： 75 Ohm unbalanced, 120 Ohm balanced, Tx loss conforms to G.703

bridge： ＞750 Ohm unbalanced, ＞1200 Ohm balanced

monitor：20 dB for input sensibility

● Signal input equalizing feature：

0—9 dB

● Signal input twittering capacity:

conforms to G.823

● Input anti-interference：conforms to G.703

● Signal structure：

Unframed

Frame (PCM30, PCM31, PCM30CRC, PCM31CRC), frame signal conforms to G.704 n×64 kbit/s structure

● Line code：HDB, or AMI

● Signal rate：2048 kbit ±50ppm

● Test pattern：False random sequence 2E6-1,2E9-1,2E11-1,2E15-1, 2E20-1, 2E23-1, conforming to ITU-T0.151, man code 8 bit

● Frequency test：For line signal and voice channel signal

● Level test：For line signal and voice channel signal

● Bit error monitoring：FAS ERR, BIT ERR, CRC-4 code error performance test, CODE ERR

● Alarm monitoring：AIS, SIG LOSS, RA, MAR, FAS LOSS, CRC LOSS, PAT LOSS, PAT SLIP

● Slip monitoring：PAT SLIP, CLK SLIP

● Time delay test：Test time delay performance

● Bit error analysis：G.821, G.826, M2100

● External clock input impedance：

Terminal：75 Ohm unbalanced, 120 Ohm balanced, Tx loss conforming to G.703

Bridge：＞750 Ohm unbalanced, ＞1200 Ohm balanced

● External clock input signal：HDB3/AMI conforming to G.703, pulse signal,＞2.5Vp-p