SAT-1XS Series E1 and Datacom Tester

Key Features

- 5 inch LCD with backlight, the largest screen in the industry

- Handheld, rugged design, easy to operation

- Particular history and real-time LED indicator design, find troubles more clearly

- Smart auto configuration feature

- Alarm and Histogram analysis for troubleshooting

- Multi-task operation at the same time

- Store 20 test results and 9 test configurations, with power-off memory

- Automatically power on/off testing by programmable timer

- Powerful PC software supports download results to PC, data analysis, report generation, printing, etc.

- Software updating

Key Functions

SAT-1AS:

Normal test

Pass through testing Audio frequency test

SAT-1BS: provides more two functions than SAT-1AS: Loop delay test

Automatic protection switching time testing (APS)

SAT-1CS: provides more three functions than SAT-1BS:

2Mbit/s line level and frequency testing

Datacom test

Co-directional 64Kbit/s test

Applications for E1

- Service-interrupted error testing

- On-line service error testing

- Framed and unframed signals generation and reception

- 2Mbit/s unframed error performance testing

- 2Mbit/s framed N×64Kbit/s channel error testing

- Bit error, coding error, frame error, CRC error and E-bit error testing

- Signal loss alarm, AIS alarm, framed remote alarm, multi-framed remote alarm, out-of- frame, and pattern loss alarm

- Frequency offset transmitting

- Voice channel signal level and frequency testing

- Pattern slip testing

- Pass through testing

- Audio frequency testing

- Loop delay test (except SAT-1AS)

- Automatic protection switching time testing (APS) (except SAT-1AS)

- Voice monitoring

- Signal state display, Voice channel content display, Voice channel busy / idle indication

- Alarm and error histogram analysis

- Time slot content analysis, drop and insert signal on each time slot

- Framed content analysis

- G. 821/G. 826/M. 2100 performance analysis

- Multi errors and alarms inserting

- Three input modes (terminating, bridging and monitoring)

- Provides two clock options (internal and picking-up);

Applications for Datacom:

- Includes V.24/V.28/RS232, V.35, V.36/RS-449, X.21, RS-485, RS422, EIA-530, EIA-530A

- SYNCH and ASYNCH testing

- DTE and DCE emulation

- Bit code testing

- Pattern slip testing

- Signal loss alarm

- Line signal frequency testing

- Loop delay testing

- Automatic protection switching time testing(APS)

- G.821, M2100 service-interrupted error testing

Applications for Co-directional 64Kbit/s:

- Service-interrupted error testing

- Bit code testing

- Pattern slip testing

- Signal loss, AIS alarm

- Line signal frequency testing

- Loop delay testing

- Automatic protection switching time testing(APS)

- G.821, M.2100 error performance testing

E1/2M Technical Specifications

- Signal input rate: 2048Kbit/s 50ppm (G.703 requirement±100ppm)

- Signal code: HDB3, AMI.

- Input jitter tolerance: up to G.823

(4)  Input Impedance

Fig. 1 Input Jitter Tolerance

Unbalance terminating: 75Ω; balance terminating: 120Ω

(5)  Signal structure

(5.1) Unframed structure

(5.2) Framed structure: PCM30, PCM31, PCM30CRC, PCM31CRC, complied with G.704 (6)

Testing pattern: 26-1, 29-1, 211-1, 215-1, 220-1, 223-1, and artificial code

(7)  Error code insertion: Bit Error, Framed Error, Pattern Slip; None, Single, Ratio 10-1~10-7.

(8)  Alarm insertion: No Signal, Frame Loss, AIS, Patten Loss

Datacom Technical Specifications

(1)   Interface type: V.24/V.28/RS232, V.35, V.36/RS-449, X.21, RS-485, RS422, EIA-530, EIA-530A

(2)   Generator

(2.1)	SYNCH mode

Clock source: internal and picking-up clock

Phase relation between clock and data: co-direction or reverse direction. Rate: 1.2, 2.4, 4.8, 9.6, 14.4, 19.2, 38.4, 48, 56(Kbps), N×64Kbps (N=1~32)

Error: ±15ppm (ppm: parts per million)

(2.2)	ASYNCH mode

Rate (bps): 50, 75, 110, 150, 200, 300, 600, 1200, 2400, 3600, 4800, 7200, 9600, 14.4K, 19.2K, 38.4K, 57.6K

Data structure: Word Length: 5, 6, 7, 8(bits)	Stop Bit: 1, 2(bits) Odd-even detection: Odd, Even, 1, 0, None.

(2.3)	Error code insertion: None, Single, or Ratio 10-1~10-7.

(3)   Receiver

(3.1)	SYNCH mode

Clock source: internal and picking up clock

Phase relation between receive clock and receive data: Co-direction or reverse direction.

Clock Rate: 2048Kbps maximum

(3.2)	ASYNCH mode

The rate and data structure are as same as the generator.

(4)   Testing pattern: 26-1, 29-1, 211-1, 215-1, 220-1, 223-1, and artificial code

Co-directional 64Kbit/s Technical Specifications

(1) Signal input rate: 64Kbit/s 50ppm(G.703 requirement ±100ppm)

(2) Input impedance: balance 120Ω, up to G.703

(3) Input jitter tolerance: up to G.823.

(4) Impedance of output interface: balance 120Ω, up to G.703

(5) Testing pattern: 26-1, 29-1, 211-1, 215-1, 220-1, 223-1, and artificial code

(6) Error code insertion: Bit Error, Pattern Slip, None, Single, Ratio 10-1~10-7.

(7) Alarm insertion: No Signal, AIS, Patter Loss

General Specifications

(1) Power supply

(1.1) Special power adapter Input: AC220V 50Hz Output: DC 9V 1.2A

(1.2) Internal rechargeable battery

4000mAh, 6V nickel-hydrogen rechargeable battery Working time: 8 hours

Charging: 8 hours at power-off state, and 12 hours at power-on state

(2) Dimension and weight L×W×H: 220×162×48 (mm)

(3) Weight: 950g

(4) Ambient parameters

Operation temperature: -10~+50℃

Storage temperature: -30~+70℃

Humidity: 5%~90%, non-condensing