SAT-20E Optical Cable Routing Locator

SAT-20E optical cable routing locator is used for locating the path of buried optic cables, measuring their depth, pinpointing cable grounding faults (sheath insulation faults), and identifying target pipelines (cables). This device is easy to operate with high intelligence, and high resistance to interference, suitable for surveying buried optic cables under various complex working conditions. It is an essential instrument for organizations involved in the operation and maintenance of buried optic cables.

Features

Test Functions: path tracing, depth measurement, sheath fault location, cable identification.

Tracking prompt: Eliminates interference from adjacent lines by calibrating the current direction, preventing tracking errors.

Depth locating: Above 0-20 meters, simultaneously displaying the route direction, current direction, and current intensity on the same interface.

Multiple frequencies detecting: 6 active detection frequencies and 2 passive detection frequencies.

Cable identification: Quickly and automatically identifies the target cable, saving engineers time and improving work efficiency.

Strong expandability(optional): External A-frame can be used for precise location of ground (sheath) faults in optical and electrical cables.

Built-in large-capacity lithium-ion battery pack, automatic shutdown due to low voltage, and automatic shutdown after a long period of inactivity.

The transmitter has an auxiliary power interface for longer operating time with an external power supply.

IP65 high-level protection.

Specifications

3.1 Receiver specification

| Function | Pipeline and cable detection (cable location tracking, direction display, depth measurement, current measurement), cable identification, and fault location. |
| --- | --- |
| Input Method | Built-in receiving coil, external flexible caliper, stethoscope, external A-frame stand(optional). |
| Receiving Frequency | Active detection：640Hz/1280Hz/8kHz/33kHz/82kHz/197kHz； Industrial frequency detection:50Hz/60Hz, industrial frequency 2/5/9 harmonic:5~15k/15~25k/25~40k |
| Pipeline Detection Mode | Broad peak method, narrow peak method, sound valley method |
| Pipeline Detection Display | Compass display mode, basic mode |
| Pipeline Route Detection Scope | Direct connection method：0-20km； Coupling method：0-10km； Inductive method：0-3km；Suitable for cables buried at a depth <2m. |
| Depth Detection | Detection range:0-19.9m Deviation: ±5%(depth<2m); ±15%(2m≤depth<8m); ±25%(8m≤depth<19.9m) |
| Voice Indication | Frequency-modulated tone that varies with signal strength. |
| Cable Identification | Intelligent identification in conjunction with flexible calipers. Loop resistance range: Direct connection method:0~20km(Loop resistance:<0Ω~8kΩ) Coupling method:0~10km(Loop resistance:<1kΩ) |
| Screen | 320*240 colorful LCD, |
| Power Supply | 7.4V DC 3000mAh rechargeable Li-ion battery; AC100-240V，50/60Hz，USB-C quick charging |
| Size& Weight | Size:680*120*277mm; Weight: 2.0Kg |
| Working Environment | Working temp.:-20℃～60℃；Storage temp.: -40℃~70℃ Humdity:95%(No-Condensing); Altitude: <4500m |

3.2 transmitter

| Function | Path tracing, cable identification, fault location |
| --- | --- |
| Output Method | Direct connection output, inductive method, caliper coupling, fault detection booster |
| Working Mode | Broad peaks; narrow peaks; valley values; historical curves; peak and valley scanning. |
| Output Frequency | 640Hz、1280Hz、8kHz、33kHz、82kHz、197kHz |
| Output Power | Max.15W,10 adjustable settings, fully automatic real-time impedance matching. |
| Output Voltage | 150Vpp MAX |
| Circuit Protection | Features overvoltage and overcurrent protection. |
| Display Content | Output voltage, output current, loop resistance. |
| Screen | 320*240 colorful LCD |
| Power Supply | 7.4VDC 6000mAh rechargeable li-ion battery, AC100-240V，50/60Hz，USB-C quick charging Auxiliary power:DC7~15V，2A |
| Size& Weight | Size: 280×220×90mm; Weight: 2.3Kg |
| Working Environment | Working temp.: -20℃～60℃；Storage temp.: -40℃~70℃ Humdity:95%(No-Condensing); Altitude: <4500m |