SAT-3F Optical laser source

SAT-3F series handheld optical laser source is mainly used for fiber optic link continuity test, visual fault locator and loss test. The output power can be adjusted by 6dB and has multiple optional wavelengths. It is widely used in fiber optic construction and maintenance, fiber optic communication, fiber optic sensing, optical CATV and other fields. It can also be used for insertion loss and splitting ratio test of passive devices such as optical splitters, circulators, optical couplers and optical switches.

Features

- Support single wavelength, and accept combined wavelength customization.

- Support output of CW and multiple modulation

- Output power is adjustable range from 6dB by 1dB step

- RJ45 sequence test

- RJ45 tracker test(optional)

- Auto shut-down time can be set.

Specification

| Models | 3F01 | 3F02 | 3F03 |
| --- | --- | --- | --- |
| Wavelength | 1310/1550±20nm | 1310/1490/1550±20nm | 850/1300±20nm |
| Output power | -5dBm |  |  |
| Power adjustable range | 0~6dB |  |  |
| Power adjustable step | 1 dB |  |  |
| Stability | ≤±0.2dB/15min (pre-heat after 15 min.) |  |  |
| Working mode | CW/270Hz/330Hz/1kHz/2kHz/ID/TWINS |  |  |
| Connector | FC/SC |  |  |
| RJ45 sequence range | ≤300m |  |  |
| RJ45 tracker | Test range ≤300m, Mode digital search; support live link search |  |  |
| General Parameters |  |  |  |
| Screen | Segment code screen |  |  |
| Power | Rechargeable li-ion 1500mAh battery |  |  |
| Auto power -off | 10 min./30 min./60 min./120 min. |  |  |
| Continuous working | ≥24h |  |  |
| Weight | 235g |  |  |
| Size | 140mmX32mmx73mm |  |  |
| Environment | Working:-10C+50℃; Storage: -40C+70℃, Humidity: 0~95% non-condensing |  |  |