SAT-4F Optical power meter

SAT-4F series optical power meter is integrated function with power meter, RJ45 sequence, flashlight, laser source, RJ45 tracker etc., it’s mainly used for continuous optical signal power test, optical link loss test and continuity test.

It is widely applied in optical cable construction and maintenance, optical fiber communication, optical cable sensing, optical CATV and other fields.

Features

- 10 wavelength power testing;

- PASS/FAIL threshold can be set.

- Wavelength ID and frequency recognition,

- RJ45 sequence test.

- Optional for RJ45 tracker and VFL.

- Auto shutdown time can be set

- Data upload and analysis

Specification

| Model | 4F01 | 4F02 |
| --- | --- | --- |
| Wavelength | 800nm-1700nm | 800nm~1700nm |
| Connector | FC/SC | FC/SC |
| Detector type | InGaAs | InGaAS |
| Power range | -70~+10dBm | -50dBm-+30dBm |
| Accuracy | ±5% |  |
| Calibrated wavelength | 850/980/1270/1300/1310/1490/1550/1577/1625/1650nm |  |
| Resolution | 0.01dB |  |
| Identified frequency | 270Hz、330Hz、1kHz、2kHz |  |
| Storage | 1000 |  |
| Custom wave | 50 |  |
| Wavelength recognition | support |  |
| VFL (Optional) | Wavelength 650±30nm; Output power ≥10mw; Mode: CW/1Hz/2Hz; Connector: universal FC/SC/ST. |  |
| RJ 45 sequency | Range ≤300m |  |
| RJ45 tracker (optional) | Range ≤300m; mode: digital tracking; live to line Support |  |
| General Parameter |  |  |
| Screen | Segment code screen |  |
| Power | Rechargeable li-ion 1500mAh battery |  |
| Auto power -off | 10 min./30 min./60 min./120 min. |  |
| Continuous working | ≥24h |  |
| Weight | 235g |  |
| Size | 140mmX32mmx73mm |  |
| Environment | Working:-10C+50℃; Storage: -40C+70℃, Humidity: 0~95% non-condensing |  |