SAT-4FWM CWDM Optical Power Meter

CWDM optical power meter is a full-featured palm-sized CWDM optical network channel detector, which is mainly used to measure and monitor the power of up to 6 or 18 CWDM channels. The test power value can be displayed in bar graph or list mode. It is specially designed and developed for the installation, acceptance and maintenance of CWDM optical networks, and has become a necessary special tool for the maintenance and installation of CWDM access networks.

Features

- Support 6 or 18 channels wavelength power test

- 3.5 inch TFT LCD, resolution 320*480.

- Single channel power information can be viewed.

- Easy to carry and easy to operate.

- Display test data in column graph or list mode.

- OPM/VFL/RJ45 sequence/cable length/cable search

- Can be charged y power bank

- User define PASS/FAIL threshold

Specification

| Model | 4FWM-6 | 4FWM-18 |
| --- | --- | --- |
| CWDM Optical power Mater |  |  |
| Detector type | InGaAS |  |
| Connector | FC/UPC |  |
| Wave range | 1270~1370nm/1271~1371nm | 1270~1610nm/1271~1611nm |
| Power | -70~+10dBm |  |
| Channel qty. | 6 | 18 |
| Channel spacing | 20nm |  |
| Channel band pass | ±6.5nm |  |
| Power resolution | 0.01dB |  |
| Power accuracy | ±0.5dB |  |
| Series test loss(dB) | ≤1.5 |  |
| Return loss | >45dB |  |
| Isolation of adjacent channel | ≥25dB |  |
| Unit | mW/μW/nW/dBm/dB (REF) |  |
| Optical Power Meter | Calibration wavelength: 850/980/1300/1310/1490/1550/1625/1650nm Power Range: -70~+10dBm, -50~+26dBm(optional) Connector: FC/PC |  |
| Visual Fault Locator | Working wavelength :650nm±20nm; universal FC/SC/ST Output power: ≥10mW; Mode: CW/1Hz/2Hz; Connector: 2.5mm |  |
| RJ45 | RJ45 length:≤300m; RJ45 sequence:≤300m;RJ45 search≤300m |  |

| General parameters |  |
| --- | --- |
| Display | 3.5 inch 320*480 color LCD |
| Storage | ≤1000 |
| Data interface | Micro USB |
| Power supply | Polymer 3.7V,4000mAh; Adapter:5VDC,2A |
| Battery life | Standby>20 hours；testing>12 hours |
| Weight | 350g |
| Size | 173mm×82mm×37mm |
| Environment | Working:-10C+50℃; Storage: -40C+70℃, Humidity: 0~95% non-condensing |