SAT-7F  PON Power Meter

SAT-7F PON power meter integrates FTTx/PON power meter, VFL, RJ45 sequence, cable length, and RJ45 tracker.

Features

- 3.5 inch TFT screen

- 1310/1490/1550nm in PON system can be tested.

- Support function VFL/ RJ45 sequence/cable length/ RJ45 tracker.

- Auto shutoff time can be set

- PASS/FAIL threshold can be set

Specification

| Wavelength | 1310±50 nm | 1490±15 nm | 1550±10 nm |
| --- | --- | --- | --- |
| Isolation 1310nm | -- | >40dB | >40dB |
| Isolation 1490nm | >40 dB | -- | >40dB |
| Isolation 1550nm | >40 dB | >40dB | -- |
| Dynamic range | -35~+10 dBm | -40~+12 dBm | -40~+25dBm |
| Uncertainty | ≤0.5dB |  |  |
| Insertion Loss | ≤1.5dB |  |  |
| Display resolution | 0.01dB |  |  |
| Connector | FC/UPC (SC can be used) |  |  |
| VFL (Optional) | Wavelength 650士30nm; Output power ≥10mw; Mode: CW/1Hz/2Hz; Connector: Universal FC/SC/ST. |  |  |
| RJ 45 sequency | Range ≤300m |  |  |
| RJ45 length | ≤300m |  |  |
| RJ45 tracker | Range ≤300m; mode:digital tracking; live to line Support |  |  |
| General parameters |  |  |  |
| Display | 3.5inch color LCD |  |  |
| Storage | ≤1000 |  |  |
| Data interface | Micro USB |  |  |
| Power supply | Polymer 3.7V,4000mAh Adapter:5VDC,2A |  |  |
| Battery life | Standby>20 hours；testing>12 hours |  |  |
| Weight | 350g |  |  |
| Size | 173mm×82mm×37mm |  |  |
| Environment | Working:-10C+50℃; Storage: -40C+70℃, Humidity: 0~95% non-condensing |  |  |