SAT-FD Wide-Range Battery Discharge

Key Features:

1. 7 inch large touch screen

2.10~600V wide-range design, can meet requirement for different voltage degree’s battery discharge testing, voltage can continuously adjust.

3. Constant current discharge function, current can continuously adjust.

4. Individual cell wireless monitor function, each module can monitor 12 single cells.

5. Support wireless battery cell monitor module and compatible with 2V/6V/12V

individual cell type.

6. Comprehensive analysis software, can remotely control discharging.

Application Scenarios

1. Daily maintenance and management of batteries
2. Inspection, acceptance, and installation of newly received batteries
3. To provide a basis for scrapping batteries
4. Quality control of battery manufacturers

Optional Function:

Internal resistance testing function

Online Compensation discharge function

Specification

| Battery Group Voltage | 24V/48V/72V/96V/110V/22V/240V/380V/480V |  |  |  |
| --- | --- | --- | --- | --- |
| Individual Cell Voltage | 2V/6V/12V |  |  |  |
| Operating Mode | Independently using, parallelly using, and remotely operating and morning. |  |  |  |
| Discharge Voltage | DC 10V~600V |  |  |  |
| Discharge Current Range | 10~19V | 19~180V | 180~300V | 300~600V |
|  | 80A | 120A | 80A | 50A |
| Battery Group Current Accuracy | 1% |  |  |  |
| Battery Group Voltage Accuracy | 0.5% |  |  |  |
| Individual Cell Voltage Accuracy | 0.2% |  |  |  |
| Individual Cell Range | 0.5V~16V (Resolution 1mv) |  |  |  |
| Discharge Time | 0~99H59Min. |  |  |  |
| Working Voltage | AC: 220V |  |  |  |
| Cooling Mode | Forced air cooling |  |  |  |
| Package | Aluminum alloy trolley case& outer wooden case |  |  |  |
| Temperature | Operating temperature: -5~50℃; storage temperature: -40~70℃ |  |  |  |
| Dimension | Accord to specific model |  |  |  |