SAT-G5 Optical Multimeter

SAT-G5 series optical power meter is integrated function with power meter, laser source, RJ45 sequence, flashlight, laser source, RJ45 tracker etc., it’s mainly used for continuous optical signal power test, optical link loss test and continuity test.

It is widely applied in optical cable construction and maintenance, optical fiber communication, optical cable sensing, optical CATV and other fields.

Features

- 10 wavelength power testing;

- Set with reference power and display relative power

- Wavelength ID and frequency recognition

- Support output of CW and multiple modulation

- Output power is adjustable range from 6dB by 1dB step

- RJ45 sequence test, RJ45 tracker test(optional)

- PASS/FAIL threshold can be set.

- Auto shut-down time can be set.

- Data upload and analysis.

Specification

| Optical Power Meter | Wavelength :800nm-1700nm Calibrated wavelength:850/980/1270/1300/1310/1490/1550/1577/1625/1650nm Frequency: 270Hz、330Hz、1kHz、2kHz Connector: FC/SC; Detector type: InGaAs Power range: -70~+10dBm/50dBm~+30dBm; Accuracy: ±5% |
| --- | --- |
| Laser Source | Wavelength :1310/1550±20nm; 1310/1490/1550±20nm Output power: -5dBm; Power range: 0~6dB; Connector: FC/SC Working mode: CW/270Hz/330Hz/1kHz/2kHz/ID/TWINS |
| VFL (Optional) | Wavelength 650士30nm; Output power ≥10mw; Mode: CW/1Hz/2Hz; Connector: Universal FC/SC/ST. |
| RJ 45 sequency | Range ≤300m |
| RJ45 tracker (optional) | Range:≤300m; mode: digital tracking; live to line :Support |
| General Parameters |  |
| Screen | Segment code screen |
| Power | Rechargeable li-ion 1500mAh battery, |
| Auto power -off | 10 min./30 min./60 min./120 min. |
| Weight | 235g |
| Size | 140mmX32mmx73mm |
| Environment | Working:-10C+50℃; Storage: -40C+70℃, Humidity: 0~95% non-condensing |